# Filler Tag Inventory

Current Historian / AVEVA tags available per filler line, with identified gaps for new data points.

# Filler 1 — Available Production Tags

Filler 1 has a full machine-level control system integration in Historian (4,900+ tags total). It runs two physical lines (sides), L1 and L2. These are the tags that I found applicable to what we want, but still some things are missing. Especially something along the lines of a liquid milk inflow totalizer.

## Carton Counts

| Tag | Description |
| --- | --- |
| Filler1.L1_Carton_Count_Infeed | Line 1 cartons at infeed (input) |
| Filler1.L2_Carton_Count_Infeed | Line 2 cartons at infeed (input) |
| Filler1.Carton_Count_Total_Infeed | Combined L1+L2 infeed total |
| Filler1.L1_Carton_Count_Good | Line 1 good/filled cartons (output) |
| Filler1.L2_Carton_Count_Good | Line 2 good/filled cartons (output) |
| Filler1.Carton_Count_Total | Combined L1+L2 good total |
| Filler1.L1_Carton_Count_Master | Line 1 master carton count |
| Filler1.L2_Carton_Count_Master | Line 2 master carton count |
| Filler1.L1_Carton_Count_Infeed_at_Stop | Line 1 infeed count at last machine stop |
| Filler1.L2_Carton_Count_Infeed_at_Stop | Line 2 infeed count at last machine stop |
| Filler1.Cartons_In_Machine | Cartons currently inside the machine |

## Job / Run Control

| Tag | Description |
| --- | --- |
| Filler1.StartJob | Start job command (discrete) |
| Filler1.Start_Button_Pressed | Physical start button press (discrete) |
| Filler1.StartPB | Start pushbutton signal (discrete) |

## Machine State & Speed

| Tag | Description |
| --- | --- |
| Filler1.Filler_State | Machine state |
| Filler1.Filler_Status | Machine status |
| Filler1.Machine_Speed_Dec | Speed decrease command |
| Filler1.Machine_Speed_Inc | Speed increase command |

## Fill Volume (setpoints — not totalizers)

| Tag | Description |
| --- | --- |
| Filler1.Fill_Max_Volume | Maximum fill volume per station |
| Filler1.Fill_Min_Volume | Minimum fill volume per station |
| Filler1.Fill_HVP_Vol | HVP fill volume |

# Fillers 2–6 — Available Tags (identical on each line)

Fillers 2–6 each have exactly 33 tags. These are all WO/MES integration tags (AVEVA). There are no hardware sensor tags or counter tags in Historian for these lines. In the last 90 days none of these tag’s values have changed, which to me means we aren’t using them.

| Tag (replace N with 2–6) | Description |
| --- | --- |
| FillerN.CurrentWO | Active work order number |
| FillerN.CurrentWOState | WO state |
| FillerN.CurrentProd | Current product being run |
| FillerN.CurrentItem | Current item number |
| FillerN.CurrentPctCmplt | Percent complete on active WO |
| FillerN.CurrentSeqNo | Current sequence number |
| FillerN.JobRunning | Job running flag (boolean) |
| FillerN.BatchSize | Planned batch size |
| FillerN.StartJob | Start job command |
| FillerN.EndJob | End job command |
| FillerN.NextWO | Next queued WO |
| FillerN.ProdQty | Produced quantity (WO output) |
| FillerN.ProdItem | Produced item |
| FillerN.ProdLot | Produced lot |
| FillerN.ProdSubLot | Produced sub-lot |
| FillerN.ProdBOMPos | Production BOM position |
| FillerN.ProdReasCd | Production reason code |
| FillerN.ConsQty | Consumed quantity (WO input/material) |
| FillerN.ConsItem | Consumed item |
| FillerN.ConsLot | Consumed lot |
| FillerN.ConsSubLot | Consumed sub-lot |
| FillerN.ConsBomPos | Consumed BOM position |
| FillerN.ConsReasCd | Consumption reason code |
| FillerN.AddProd | Add production record command |
| FillerN.AddProdAPI | Add production (API trigger) |
| FillerN.AddCons | Add consumption record command |
| FillerN.AddConsAPI | Add consumption (API trigger) |
| FillerN.GetJobList | Fetch available job list |
| FillerN.GetCurrentJobInfo | Fetch current job info |
| FillerN.GetJobBom | Fetch job BOM |
| FillerN.GetSubLot | Fetch sub-lot info |
| FillerN.FromEntName | Source entity name |
| FillerN.ToEntName | Destination entity name |

# Requested New Tags / Data Points

The following data points do not currently exist in Historian and would need to be created or exposed by the automation system.

| Desired Metric | Filler 1 | Fillers 2–6 | Notes |
| --- | --- | --- | --- |
| Job kickoff counter (increments each run start) | Gap: StartJob signal exists but no counter tag | Gap: No tag exists | Needs a new accumulator tag per line |
| Reject / waste carton count | Partial: Derivable as Infeed − Good but not exposed | Gap: No tag exists | Expose calculated reject count as a tag |
| Product volume totalizer (gallons, running total) | Gap: Fill volume setpoints exist — no flow accumulator | Gap: No hardware sensor or totalizer | Requires flow meter or volumetric totalizer per line |
| Carton infeed count | Exists: L1/L2_Carton_ Count_Infeed | Gap: No tag exists | Needed on Fillers 2–6 |
| Carton good/filled count | Exists: L1/L2_Carton_ Count_Good | Gap: No tag exists | Real-time sensor count needed on Fillers 2–6 |

# Cappers— Available Tags

Note: tags in Historian use the naming Capper3, Capper4, Capper5, Capper6 — there is no Capper1 or Capper2.

Each capper has 24 WO/MES integration tags. Unlike the fillers, the cappers have NO consumption tags (AddCons, ConsQty, ConsItem, etc.) — cap usage is not tracked in Historian at all. All tags are confirmed stale: values have not changed in 90 days despite lines running.

## Capper 3, 4, 5, 6 — WO Tags (identical on each)

| Tag (replace N with 3-6) | Description |
| --- | --- |
| CapperN.CurrentWO | Active work order number |
| CapperN.CurrentWOState | WO state |
| CapperN.CurrentProd | Current product being run |
| CapperN.CurrentItem | Current item number |
| CapperN.CurrentPctCmplt | Percent complete on active WO |
| CapperN.CurrentSeqNo | Current sequence number |
| CapperN.JobRunning | Job running flag (boolean) |
| CapperN.BatchSize | Planned batch size |
| CapperN.StartJob | Start job command |
| CapperN.EndJob | End job command |
| CapperN.NextWO | Next queued WO |
| CapperN.ProdQty | Produced quantity (WO output) |
| CapperN.ProdItem | Produced item |
| CapperN.ProdLot | Produced lot |
| CapperN.ProdSubLot | Produced sub-lot |
| CapperN.ProdBOMPos | Production BOM position |
| CapperN.ProdReasCd | Production reason code |
| CapperN.AddProd | Add production record command |
| CapperN.AddProdAPI | Add production (API trigger) |
| CapperN.GetJobList | Fetch available job list |
| CapperN.GetCurrentJobInfo | Fetch current job info |
| CapperN.GetJobBom | Fetch job BOM |
| CapperN.GetSubLot | Fetch sub-lot info |
| CapperN.ToEntName | Destination entity name |

## Capper Gaps

| Desired Metric | Status | Notes |
| --- | --- | --- |
| Kickoff events | Stale — StartJob signal exists but value never changes | Need working WO integration or new counter tag |
| Uncapped cartons input | Missing — no tag exists | New tag required |
| Cartons capped output | Stale — ProdQty exists but never updates | Need working WO integration or new counter tag |
| Caps used (consumption) | Missing — consumption tags do not exist on cappers | New tags required; AddCons/ConsQty need to be added to capper WO templates |

# Case Packers — Available Tags

CasePacker1 through CasePacker6 all have the standard 24-tag WO/MES integration set (same as fillers and cappers, same stale/non-functional status). However, CasePacker5 and CasePacker6 also have dedicated machine-level counter tags that are a different story.

## Machine Counter Tags — Lines 5 & 6 only

| Tag | Description | Status | Last Value | Last Changed |
| --- | --- | --- | --- | --- |
| CasePacker5_CaseCounts | L5 Case Counts | Live — actively changing | 3,845 | 5/8/2026 9:46 AM |
| CasePacker5_LifeTimeCounts | L5 Case Life Time Counts | Live — actively changing | 1,368,946 | 5/8/2026 9:46 AM |
| CasePacker6_CaseCounts | CasePacker6 Counts | Stale since 5/5/2026 | 7,077 | 5/5/2026 8:17 AM |
| CasePacker6_LifeTimeCounts | L6 Life Time Counts | Stale since 5/5/2026 | 869,500 | 5/5/2026 8:17 AM |

Lines 1-4 have no machine-level counter tags — only the stale WO tags.

## Case Packer Gaps

| Desired Metric | Lines 5 & 6 | Lines 1-4 | Notes |
| --- | --- | --- | --- |
| Kickoff events | Stale WO tag only | Stale WO tag only | New counter tag needed on all lines |
| Cases in / out | Live via CaseCounts tag | Missing | Lines 1-4 need machine-level counter tags like lines 5 & 6 |
| Cases used total | Live via LifeTimeCounts tag | Missing | Lines 1-4 need equivalent tags |

# Palletizers — Available Tags

The only data point needed from palletizers is a running case infeed count — how many cases have been fed into each palletizer over time. Two tag systems exist. The standard WO/MES tags (Palletizer1-6) are stale like all other equipment. The Pack_PackN_PallN hardware I/O tags are more promising but have mixed status.

## Hardware I/O Tags — Relevant to Case Infeed

| Tag | Description | Status | Notes |
| --- | --- | --- | --- |
| Pack_PackN_PallN_IO.PalletCasesPrev | Cases from last completed pallet — live and changing on lines 1/3/4/5, values in the millions suggesting it is a cumulative total. Needs automation confirmation. | Partial — confirm if cumulative | Most likely the case infeed counter we need |
| Pack_PackN_PallN_IO.PalletCases | Cases on current in-progress pallet | Broken — stuck at 0 on all lines | Not usable in current state |

## PalletCasesPrev — Line by Line Status

| Line | PalletCasesPrev Value | Last Updated | Notes |
| --- | --- | --- | --- |
| Line 1 | Live — 5,922,355 | 5/8/2026 2:50 AM | Changing — scale confirms cumulative, not per-pallet |
| Line 2 | Stale — 644,951 | 5/2/2026 | Not updating — needs investigation |
| Line 3 | Live — 1,217,200 | 5/8/2026 9:42 AM | Changing |
| Line 4 | Live — 526,474 | 5/8/2026 9:42 AM | Changing |
| Line 5 | Live — 1,419,685 | 5/8/2026 9:47 AM | Changing |
| Line 6 | Missing — null | Last seen 2/7/2026 | Palletizer 6 completely disconnected from Historian |

## Palletizer Gaps

| Desired Metric | Status | Notes |
| --- | --- | --- |
| Case infeed count | Partial — PalletCasesPrev live on lines 1/3/4/5, broken on line 2, disconnected on line 6 | Confirm with automation that PalletCasesPrev is cumulative; fix line 2; reconnect line 6 |

## Filler 1 — Cap Monitoring Tags (existing)

Filler 1 exposes cap-related monitoring tags in Historian. These confirm the machine is capping, but there is no cap counter — no tag accumulates how many caps have been applied. The cap detect photoeye exists and could be used as the trigger for a new counter tag.

| Tag | Description | Type |
| --- | --- | --- |
| Filler1.g_b_Cap_Detect_Eye | Cap detect photoeye — fires per carton | Discrete |
| Filler1.Flt_L1_No_Caps | Fault: Line 1 detected 3 cartons in a row with no cap | Discrete |
| Filler1.Flt_L2_No_Caps | Fault: Line 2 detected 3 cartons in a row with no cap | Discrete |
| Filler1.Spout_Feeder_Bowl_Spout_Request | Feeder bowl requesting a cap | Discrete |
| Filler1.g_b_Accel_Air_Enable | Air blowing caps to the tracks | Discrete |
| Filler1.Spout_Cap_Track_Air_Jet_Enable | Cap track air jet enable | Discrete |
| Filler1.g_b_Cap_Cleanout | Cap cleanout command | Discrete |
| Filler1.g_b_Cap_Clean_Out_IP | Cap cleanout in progress | Discrete |
| Filler1.Spout_Clean_Out_Time_Remaining | Cap cleanout time remaining | Analog |
| Filler1.g_i_Load_Pocket | Spout cap loading pocket position | Analog |
| Filler1.L1_Spout_Torque_Max | Line 1 spout application torque max | Analog |
| Filler1.L2_Spout_Torque_Max | Line 2 spout application torque max | Analog |
| Filler1.g_b_Avg_Torque | Average torque fault | Discrete |
| Filler1.g_b_Avg_Torque_Warn | Average torque warning | Discrete |

## Filler 1 — Capper Gaps

| Desired Metric | Status | Notes |
| --- | --- | --- |
| Kickoff count | Partial — StartJob exists | Same as filler; no dedicated counter |
| Cartons capped out | Partial — derivable from L1/L2_Carton_Count_Good | Good carton count = capped carton count on this machine |
| Caps used count | No tag — cap detect eye exists | Add counter tag incrementing on g_b_Cap_Detect_Eye rising edge |
| Uncapped / rejected | Partial — Flt_L1/L2_No_Caps fault exists | Fault fires at 3-in-a-row threshold, not a per-carton reject count |

## Filler 2 — No Cap Tags Exist

Filler 2 is also a combined filler/capper, but unlike Filler 1 it has zero machine-level tags in Historian — only the 33 stale WO/MES integration tags. There is no cap detect eye, no torque monitoring, no fault tags, and no cap counter of any kind. The hardware integration for Filler 2 needs to be built from scratch, equivalent to what Filler 1 already has.

| Desired Metric | Status | Notes |
| --- | --- | --- |
| Kickoff count | Stale WO tag only | StartJob exists but never changes |
| Cartons capped out | No tag | No carton count tags of any kind |
| Caps used count | No tag | No cap detect eye to build from |
| Uncapped / rejected | No tag | No fault tags either |